Modelling of the external signals of the vagus.

Oliver Case1(0009-0007-4499-0682)*, Sarah N. Garfinkel2(0000-0002-5961-1012), Pier David Lambiase1, Gareth R Barnes3(0000-0002-5396-7712)

1 UCL Centre for Neurocardiology, Institute of Cardiovascular Science, University College London, London, United Kingdom

2 Institute of Cognitive Neuroscience, University College London, London, United Kingdom

3 Department of Imaging Neuroscience, UCL Queen Square Institute of Neurology, University College London, London, United Kingdom.

*Corresponding Author:

Oliver Case, UCL Centre for Neurocardiology, Institute of Cardiovascular Science, University College London, London, United Kingdom.

E. oliver.case.25@ucl.ac.uk

Abstract

xxx

Keywords: xxx

Introduction

xxx

Anatomical inputs and tissue surface preparation.

Surface meshes were derived from the BodyParts3D open atlas (RIKEN/University of Tokyo): one whole-body skin surface, 74 individual bones spanning the cervical-thoracic skeleton, and the left and right cervical vagus trunks. Per-tissue surfaces were processed by a layered watertightening pipeline (vagus_fm.geometry.builder) that applied, in order, (i) cheap trimesh repair (vertex merging, removal of degenerate and duplicate faces, normal repair), (ii) boolean union of spatially overlapping connected components, (iii) voxel shrinkwrap reconstruction, and (iv) pymeshfix as a final hole-closer. The shrinkwrap stage rasterised each compartment at a per-tissue voxel pitch (0.5–3 mm), applied morphological closing and exterior flood-fill, re-extracted the surface via marching cubes, and finished with quadric decimation and Taubin smoothing. Each compartment was gated on watertightness, winding consistency, and Euler number = 2; the build raised rather than persisted any compartment failing those tests.

Tetrahedral FEM construction.

The four-tissue tetrahedral mesh was built on a single global voxel grid (3 mm pitch, 50 mm padding) covering all compartments (vagus_fm.fem.cgal_builder). Each tissue was rasterised by the strategy appropriate to its input: convex-hull union per bone clipped to the skin envelope; ray-traced + flood-fill for the watertight vagus tubes (with optional dilation to reach a target diameter); and surface-sample splatting + morphological closing + exterior flood for the skin, which accommodates anatomical openings (mouth, nostrils) that defeat naive interior fills. Tissues were composited into a single labelled image, painted outermost-first so inner labels overwrote outer; CGAL Mesh_3 was then invoked through iso2mesh.cgalv2m (radbound 2.5, maximum element volume 20 voxel³). Joe-Liu mean-ratio quality was computed per element (vagus_fm.mesh.quality) and gated at min ≥ 0.05; bounding-box extent was checked to confirm millimetre units. Final mesh: 209,927 nodes, 789,342 elements, four tissues (vagus_left, vagus_right, bone, skin); minimum quality 0.092, fifth percentile 0.301, mean 0.524.

OPM and surface-electrode arrays.

A triaxial OPM array was generated by inward-radial cylindrical raycasting against the skin surface (vagus_fm.sensors.triaxial). Cylinder radius was 0.75× max(X-extent, Y-extent); Z-spacing and angular spacing were matched at 30 mm. Glancing hits (>90° between ray and outward normal) were rejected. Each surface hit emitted one position offset by a configurable stand-off along the outward normal, with three orthogonal coil orientations (radial + two SVD-derived tangents), in canonical FieldTrip grad layout: 8,190 channels (2,730 positions × 3 axes).

Forward solves with DUNEuro.

Both leadfields were computed by DUNEuro 2.10 (built from source via cluster/build_duneuro.sh; Python bindings via duneuropy). The volume conductor was configured in mm-mode (σ in S/mm) under a single conductivity assignment: vagus 0.30, bone 0.0042, skin 0.43 S/m (IT'IS Database v4.1; isotropic, frequency-independent). The MEG path (vagus_fm.forward.solve) attached OPMs as point coils with their tangent orientations and used DUNEuro's transfer-matrix API: computeMEGTransferMatrix followed by applyMEGTransfer for three orthogonal moments per source. Solver settings were CG, reduction 10⁻¹⁰, SIPG scheme, integration-order add 5, with subtract_mean and post_process_meg enabled. The EEG path (vagus_fm.forward.eeg) attached electrodes with the closest-subentity-centre realisation (codims=1) and used computeEEGTransferMatrix + applyEEGTransfer; surface potentials were common-average referenced by per-column subtraction across electrodes. Source positions were sampled at 5 mm spacing along the vagus polyline by Z-slab averaging of tet centroids of vagus_left (vagus_fm.sources.vagus.vagus_sources), yielding 87 cervical sources by default.

Calibration to absolute units.

DUNEuro's mm-mode raw output is dimensionally ambiguous, so both modalities were calibrated against analytic gold standards. For MEG, the conversion factor (×10⁶ from DUNEuro mm-mode to fT per nA·m source moment) was verified against the Sarvas closed-form solution and against MNE-Python's _do_sphere_fieldreference implementation to machine precision (rtol = 10⁻¹²; tests/test_analytic_sphere.py, four parametrised dipole orientations). For EEG, no a-priori dimensional analysis was reliable, so we calibrated empirically (vagus_fm.analysis.sphere_calibration): a homogeneous-tissue sphere FEM (R = 100 mm, σ = 0.43 S/m, voxel pitch 4 mm, ~2,700 elements after CGAL meshing) was meshed identically to the production pipeline; 200 surface electrodes were placed on the sphere via Fibonacci lattice; the EEG forward was computed with a unit dipole at 50 mm depth; and the per-(electrode, moment) ratio between the Berg–Scherg analytic series and the DUNEuro raw output was taken as the conversion factor. We adopted the median over (electrode, moment) pairs to be robust to near-zero analytic values. The headline factor was 0.622 (geomean 0.738). A 27-case robustness sweep across radius {60, 100, 140} mm × source depth {0.3, 0.5, 0.7}·R × conductivity {0.1, 0.43, 1.0} S/m gave median 0.671, std 0.046, CV = 6.8%; the factor was σ-invariant by construction (DUNEuro's σ-scaling and Berg–Scherg's 1/σ dependence cancel exactly). The production constant EEG_CALIBRATION_FACTOR = 0.622 is set in vagus_fm/forward/eeg.py.

Sarvas vs full-FEM benchmark.

The cervical-vagus geometry was approximated by the concentric-circle parameters of Bu et al. (source–axis distance 40 mm; sensor–axis distance 58.5 mm including the QuSpin 6.5 mm stand-off). Critically, we used a moving sphere centre per source — at the same Z as each source, on the cervical axis — rather than a single fixed sphere for the whole cervical column, which would inflate Sarvas amplitudes for sources far from the reference Z (vagus_fm.analysis.sarvas_compare). The cervical-axis (X, Y) was estimated from a 2:1 weighted average of bone tet centroids and source-polyline XY within the source Z-band. For each (source, coil) pair, the FEM leadfield was projected onto the local vagus tangent (longitudinal moment) and the Sarvas analytic prediction was computed with the same dipole orientation and moment. Statistics were reported both array-wide and restricted to a literature band (40 ± 30 mm source–axis, 58.5 ± 30 mm coil–axis distance). 95% confidence intervals on the peak FEM/Sarvas ratio were obtained by non-parametric bootstrap (B = 2,000 resamples) over (source, coil) pairs.

Source model and physiological scenarios.

Per-fibre dipole moments were computed via the Hämäläinen formulation Q = π·d²·σ_in·ΔV/4, with σ_in = 1.0 S/m (canonical brain-MEG convention; switchable to the peripheral-axoplasm value 0.35 S/m), and action-potential amplitudes ΔV = 70 mV (myelinated A) or 80 mV (unmyelinated C). Conduction velocity followed Hursh's law (CV ≈ 6 m·s⁻¹·µm⁻¹) for myelinated fibres and 0.5 m·s⁻¹ for unmyelinated. Two physiological scenarios were implemented (vagus_fm.physiology.scenarios): (i) cardiac-locked baroreceptor activity at 70 bpm (1.17 Hz), ~200 A-fibres per burst (lognormal diameter centred on 8 µm) at 150 ms post-R-wave with ±5 ms jitter; (ii) slow / deep breathing at 6 breaths·min⁻¹ (0.10 Hz), with a phasic rapidly-adapting receptor burst of ~600 mixed pulmonary fibres at inspiration onset and a slowly-adapting tonic train of ~80 fibres at 50 Hz over 45% of the cycle. The simulator (vagus_fm.physiology.simulate) generated per-channel time series at fs = 30 kHz by superposing biphasic-Gaussian compound action potentials (σ = 0.5 ms) at the cervical hot-spot. A propagating-CAP control benchmark traversed the full vagus polyline at fibre-diameter-dependent CV.

Noise floors and detectability.

Per-channel noise was modelled as quadrature-summed white noise integrated over a 1 kHz analysis bandwidth (vagus_fm.analysis.snr.compute_noise_floors). For OPM-MEG, σ_n = 7 fT/√Hz × √1000 Hz = 221 fT (QuSpin Gen-3, QZFM-3); for HD-EMG, the amplifier term (0.5 µV/√Hz) was summed in quadrature with the Johnson voltage noise of the skin contact, v_n = √(4 k_B T R), at body temperature T = 310.15 K and R = 50 kΩ (≈ 0.029 µV/√Hz; negligible at PEDOT:PSS impedances), giving σ_n ≈ 15.8 µV. Per-source signal amplitude was the channel-RMS of the orthogonal-moment-averaged leadfield; trials-to-detect at the Rose criterion (SNR = 3) was inverted from SNR ∝ √N for white uncorrelated noise. We document, in figure captions, that spatially-correlated environmental MEG noise inflates trials-to-detect by 1–10× and that LCMV/SAM beamforming reduces it by a comparable factor.

Data Availability

xxx

Funding Statement

This research is funded by the British Heart Foundation (BHF), Grant code: SI/F/24/21170016 and RG/F/25/110148, and the Discovery Research Platform for Naturalistic Neuroimaging, funded by Wellcome Grant code: 226793/Z/22/Z.

Conflict of Interest

The authors declare that the research was conducted in the absence of any commercial or financial relationships that could be construed as a potential conflict of interest.

Ethics Statement

Not applicable, as this article is a review that does not involve original research with human participants or animals.

CRediT Author Statement

Oliver Case: Conceptualisation; Writing – original draft; Writing – review & editing; Visualisation. Sarah N. Garfinkel: Conceptualisation; Writing – review & editing; Supervision. Gareth R. Barnes: Conceptualisation; Writing – review & editing; Supervision; Funding acquisition. Pier Lambiase: Writing – review & editing; Funding acquisition.

References